| 컴퓨터구조 결 과 보 고 서 (Result report) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Major | Student ID | Grade | Name | Experiment date |
| 융합전자공학부 | 2015036580 | 3 | 김수영 | 2021-06-24 |
| Title | Basic Pipeline 과제 |  |  |  |

1. 전체 코드 첨부

(디버깅을 위한 output은 주석 처리되어 있음)

`timescale 1ns / 1ps

//////////////////////////////////////////////////////////////////////////////////

// Company: Hanyang Univ.

// Engineer: Kim Soo Young

//

// Create Date:    13:14:17 06/02/2021

// Design Name:

// Module Name:    single_cycle_main

// Project Name:

// Target Devices:

// Tool versions:

// Description:

//

// Dependencies:

//

// Revision:

// Revision 0.01 - File Created

// Additional Comments:

//

//////////////////////////////////////////////////////////////////////////////////

module basic_pipeline (clk, reset, result

// PC_now, Instruction_now,

// Rs_now, Rt_now, ALU_input_1, ALU_input_2, immi_Shifted, PC_4,

// load_data, Beq_address, Write_Register,

// debug_flag_2, Debug_RegWrite, Debug_Write_Data

);

input clk, reset;

output [31:0] result; // ALU result

// Debugging Outputs

// output [31:0] PC_now; // current PC value Debugging,

// output [31:0] Instruction_now; // Fetch stage instruction

// output [31:0] Rs_now, Rt_now; // output from Register File

// output [31:0] ALU_input_1, ALU_input_2; // ALU inputs

// output [31:0] PC_4, Beq_address, immi_Shifted; // for Branch Address Debugging

// output [4:0] Write_Register; // Rd value

// output [31:0] load_data; // Data_memory_read

// output [31:0] debug_flag_2, Debug_Write_Data; // ALU_result in MEM stage, Write_Data for Register File

// output Debug_RegWrite; // RegWrite Op Flag

// IF stage

wire [31:0] PC_in;

wire [31:0] PC_out;

wire [31:0] PC_plus4;

wire [31:0] IF_instruction;

// ID stage

wire [31:0] ID_PC_plus4;

wire [31:0] ID_instruction;

wire [31:0] Read_data_1, Read_data_2;

wire [31:0] sign_extended_immi;

wire ALUSrc, RegWrite;

wire RegDst, Jump, Branch;

wire MemRead, MemtoReg, MemWrite;

wire [1:0] ALUOp;

// EX stage

wire [31:0] EX_PC_plus4;

wire [31:0] EX_instruction;

wire [31:0] EX_Read_data_1, EX_Read_data_2;

wire [31:0] EX_sign_extended_immi;

wire EX_ALUSrc, EX_RegWrite;

wire EX_RegDst, EX_Jump, EX_Branch;

wire EX_MemRead, EX_MemtoReg, EX_MemWrite;

wire [1:0] EX_ALUOp;

wire [4:0] EX_Write_Register;

wire [3:0] operation_code;

wire [31:0] Branch_addr;

wire [31:0] Shifted_immi;

wire [31:0] ALU_input_B;

wire [31:0] ALU_result;

wire ALU_zero;

// MEM stage

wire [31:0] MEM_ALU_result, MEM_Read_data_2;

wire [4:0] MEM_Write_Register;

wire [31:0] Data_memory_read;

wire [31:0] MEM_Branch_addr;

wire MEM_RegWrite, MEM_MemtoReg, MEM_Branch;

wire MEM_MemRead, MEM_MemWrite, MEM_Jump;

wire MEM_ALU_zero;

wire PCSrc;

// WB stage

wire [31:0] WB_Data_memory_read, WB_ALU_result;

wire [4:0] WB_Write_Register;

wire [31:0] WB_Write_Data;

wire WB_RegWrite, WB_MemtoReg;

//////////////////////////////////

//// Instruction Fetch stage /////

//////////////////////////////////

N_bit_MUX #(32) pc_mux (.input0(PC_plus4), .input1(MEM_Branch_addr),

.mux_out(PC_in), .control(PCSrc));

Program_Counter program_counter (.clk(clk), .reset(reset),

.PC_in(PC_in), .PC_out(PC_out));

Instruction_Memory instruction_memory (.address(PC_out),

.instruction(IF_instruction), .reset(reset));

ALU_add_only pc_add_4 (.input1(PC_out), .input2(32'b0100), .add_out(PC_plus4));

IF_ID_Stage_Reg IF_ID_Stage_Unit (.clk(clk), .reset(reset),

.PC_plus4_in(PC_plus4), .PC_plus4_out(ID_PC_plus4),

.instruction_in(IF_instruction), .instruction_out(ID_instruction));

// assign PC_now = PC_out;

// assign Instruction_now = IF_instruction;

//////////////////////////////////

//// Instruction Decode stage ////

//////////////////////////////////

// assign Debug_RegWrite = WB_RegWrite;

// assign Debug_Write_Data = WB_Write_Data;

Register_File regfile_Unit (.clk(clk), .reset(reset),

.Read_Register_1(ID_instruction[25:21]),

.Read_Register_2(ID_instruction[20:16]),

.Write_Register(WB_Write_Register), .Write_Data(WB_Write_Data),

.Read_Data_1(Read_data_1), .Read_Data_2(Read_data_2), .RegWrite(WB_RegWrite));

Sign_Extension immi_sign_extension (.input_16(ID_instruction[15:0]), .output_32(sign_extended_immi));

Control control_unit (.OpCode(ID_instruction[31:26]),

.RegDst(RegDst), .Jump(Jump), .Branch(Branch),

.MemRead(MemRead), .MemtoReg(MemtoReg), .ALUOp(ALUOp),

.MemWrite(MemWrite), .ALUSrc(ALUSrc), .RegWrite(RegWrite));

ID_EX_Stage_Reg ID_EX_Stage_Unit (.clk(clk), .reset(reset),

.RegWrite_in(RegWrite), .RegWrite_out(EX_RegWrite),

.MemtoReg_in(MemtoReg), .MemtoReg_out(EX_MemtoReg),

.Branch_in(Branch), .Branch_out(EX_Branch),

.MemRead_in(MemRead), .MemRead_out(EX_MemRead),

.MemWrite_in(MemWrite), .MemWrite_out(EX_MemWrite),

.Jump_in(Jump), .Jump_out(EX_Jump),

.RegDst_in(RegDst), .RegDst_out(EX_RegDst),

.ALUSrc_in(ALUSrc), .ALUSrc_out(EX_ALUSrc),

.ALUOp_in(ALUOp), .ALUOp_out(EX_ALUOp),

.PC_plus4_in(ID_PC_plus4), .PC_plus4_out(EX_PC_plus4),

.read_data_1_in(Read_data_1), .read_data_1_out(EX_Read_data_1),

.read_data_2_in(Read_data_2), .read_data_2_out(EX_Read_data_2),

.sign_extended_immi_in(sign_extended_immi), .sign_extended_immi_out(EX_sign_extended_immi),

.instruction_in(ID_instruction), .instruction_out(EX_instruction));

// assign Rs_now = Read_data_1;

// assign Rt_now = Read_data_2;

//////////////////////////////////

///////   Execute  stage  ////////

//////////////////////////////////

N_bit_MUX #(5) write_reg_mux (.input0(EX_instruction[20:16]), .input1(EX_instruction[15:11]),

.mux_out(EX_Write_Register), .control(EX_RegDst));

ALU_Control alu_control_unit (.ALUOp(EX_ALUOp), .f_code(EX_instruction[5:0]), .operation_code(operation_code));

N_bit_MUX #(32) alu_input_mux (.input0(EX_Read_data_2), .input1(EX_sign_extended_immi),

.mux_out(ALU_input_B), .control(EX_ALUSrc));

ALU alu_unit (.input1(EX_Read_data_1), .input2(ALU_input_B),

.alu_out(ALU_result), .zero(ALU_zero), .control(operation_code));

assign Shifted_immi = { EX_sign_extended_immi[29:0], 2'b00 };

ALU_add_only alu_add_only_unit (.input1(EX_PC_plus4), .input2(Shifted_immi), .add_out(Branch_addr));

EX_MEM_Stage_Reg EX_MEM_Stage_Unit ( .clk(clk), .reset(reset),

.RegWrite_in(EX_RegWrite), .RegWrite_out(MEM_RegWrite),

.MemtoReg_in(EX_MemtoReg), .MemtoReg_out(MEM_MemtoReg),

.Branch_in(EX_Branch), .Branch_out(MEM_Branch),

.MemRead_in(EX_MemRead), .MemRead_out(MEM_MemRead),

.MemWrite_in(EX_MemWrite),.MemWrite_out(MEM_MemWrite),

.Jump_in(EX_Jump), .Jump_out(MEM_Jump),

.Branch_addr_in(Branch_addr), .Branch_addr_out(MEM_Branch_addr),

.ALU_zero_in(ALU_zero), .ALU_zero_out(MEM_ALU_zero),

.ALU_result_in(ALU_result), .ALU_result_out(MEM_ALU_result),

.Read_data_2_in(EX_Read_data_2), .Read_data_2_out(MEM_Read_data_2),

.RegisterRd_in(EX_Write_Register), .RegisterRd_out(MEM_Write_Register));

// assign PC_4 = EX_PC_plus4;

// assign immi_Shifted = Shifted_immi;

// assign ALU_input_1 = EX_Read_data_1;

// assign ALU_input_2 = ALU_input_B;

// assign Beq_address = Branch_addr;

assign result = ALU_result;

//////////////////////////////////

///////   Memory  stage  /////////

//////////////////////////////////

Data_Memory data_memory_unit (.clk(clk), .reset(reset), .MemAddr(MEM_ALU_result[7:0]),

.Write_Data(MEM_Read_data_2), .Read_Data(Data_memory_read),

.MemRead(MEM_MemRead), .MemWrite(MEM_MemWrite));

and (PCSrc, MEM_Branch, MEM_ALU_zero);

// assign debug_flag_2 = MEM_ALU_result;

// assign load_data = Data_memory_read;

MEM_WB_Stage_Reg MEM_WB_Stage_Unit (.clk(clk), .reset(reset),

.RegWrite_in(MEM_RegWrite), .RegWrite_out(WB_RegWrite),

.MemtoReg_in(MEM_MemtoReg), .MemtoReg_out(WB_MemtoReg),

.Data_memory_read_in(Data_memory_read), .Data_memory_read_out(WB_Data_memory_read),

.ALU_result_in(MEM_ALU_result), .ALU_result_out(WB_ALU_result),

.Write_Register_in(MEM_Write_Register), .Write_Register_out(WB_Write_Register));

//////////////////////////////////

///   Write Back stage      //////

//////////////////////////////////

N_bit_MUX #(32) write_data_mux (.input0(WB_ALU_result), .input1(WB_Data_memory_read),

.mux_out(WB_Write_Data), .control(WB_MemtoReg));

// assign Write_Register = WB_Write_Register;

endmodule

// IF/ID stage register

module IF_ID_Stage_Reg (clk, reset, PC_plus4_in, PC_plus4_out,

instruction_in, instruction_out);

input clk, reset;

input [31:0] PC_plus4_in, instruction_in;

output reg [31:0] PC_plus4_out, instruction_out;

always @(posedge clk or negedge reset) begin

if (!reset) begin

PC_plus4_out <= 32'b0;

instruction_out <= 32'b0;

end

else begin

PC_plus4_out <= PC_plus4_in;

instruction_out <= instruction_in;

end

end

endmodule

// ID/EX stage register

module ID_EX_Stage_Reg (clk, reset, RegWrite_in, RegWrite_out, MemtoReg_in, MemtoReg_out,

Branch_in, Branch_out, MemRead_in, MemRead_out, MemWrite_in, MemWrite_out,

Jump_in, Jump_out, RegDst_in, RegDst_out, ALUSrc_in, ALUSrc_out, ALUOp_in, ALUOp_out,

PC_plus4_in, PC_plus4_out, read_data_1_in, read_data_1_out,

read_data_2_in, read_data_2_out,

sign_extended_immi_in, sign_extended_immi_out,

instruction_in, instruction_out);

// WB control signal

input RegWrite_in, MemtoReg_in;

output reg RegWrite_out, MemtoReg_out;

// MEM control signal

input Branch_in, MemRead_in, MemWrite_in, Jump_in;

output reg Branch_out, MemRead_out, MemWrite_out, Jump_out;

// EX control signal

input RegDst_in, ALUSrc_in;

output reg RegDst_out, ALUSrc_out;

input [1:0] ALUOp_in;

output reg [1:0] ALUOp_out;

// addr content

input [31:0] PC_plus4_in;

output reg [31:0] PC_plus4_out;

// data content

input [31:0] read_data_1_in, read_data_2_in, sign_extended_immi_in;

output reg [31:0] read_data_1_out, read_data_2_out, sign_extended_immi_out;

// reg content

input [31:0] instruction_in;

output reg [31:0] instruction_out;

// general signal

input clk, reset;

always @(posedge clk or negedge reset) begin

if (!reset) begin

RegWrite_out <= 1'b0; MemtoReg_out <= 1'b0;

Branch_out <= 1'b0; MemRead_out <= 1'b0;

MemWrite_out <= 1'b0; Jump_out <= 1'b0;

RegDst_out <= 1'b0; ALUSrc_out <= 1'b0;

ALUOp_out <= 2'b0;

PC_plus4_out <= 32'b0;

read_data_1_out <= 32'b0; read_data_2_out <= 32'b0;

sign_extended_immi_out <= 32'b0;

instruction_out <= 32'b0;

end

else begin

RegWrite_out <= RegWrite_in; MemtoReg_out <= MemtoReg_in;

Branch_out <= Branch_in; MemRead_out <= MemRead_in;

MemWrite_out <= MemWrite_in; Jump_out <= Jump_in;

RegDst_out <= RegDst_in; ALUSrc_out <= ALUSrc_in;

ALUOp_out <= ALUOp_in; PC_plus4_out <= PC_plus4_in;

sign_extended_immi_out <= sign_extended_immi_in;

read_data_1_out <= read_data_1_in;

read_data_2_out <= read_data_2_in;

instruction_out <= instruction_in;

end

end

endmodule

// EX/MEM stage register

module EX_MEM_Stage_Reg (clk, reset,

RegWrite_in, RegWrite_out, MemtoReg_in, MemtoReg_out,

Branch_in, Branch_out, MemRead_in, MemRead_out,

MemWrite_in, MemWrite_out, Jump_in, Jump_out,

Branch_addr_in, Branch_addr_out, ALU_zero_in, ALU_zero_out,

ALU_result_in, ALU_result_out, Read_data_2_in, Read_data_2_out,

RegisterRd_in, RegisterRd_out);

// WB control signal

input RegWrite_in, MemtoReg_in;

output reg RegWrite_out, MemtoReg_out;

// MEM control signal

input Branch_in, MemRead_in, MemWrite_in, Jump_in;

output reg Branch_out, MemRead_out, MemWrite_out, Jump_out;

// addr content

input [31:0] Branch_addr_in;

output reg [31:0] Branch_addr_out;

// data content

input ALU_zero_in;

output reg ALU_zero_out;

// results

input [31:0] ALU_result_in, Read_data_2_in;

output reg [31:0] ALU_result_out, Read_data_2_out;

// registers

input [4:0] RegisterRd_in;

output reg [4:0] RegisterRd_out;

// general signal

input clk, reset;

always @(posedge clk or negedge reset) begin

if (!reset) begin

RegWrite_out <= 1'b0; MemtoReg_out <= 1'b0;

Branch_out <= 1'b0; MemRead_out <= 1'b0;

MemWrite_out <= 1'b0; Jump_out <= 1'b0;

Branch_addr_out <= 32'b0; ALU_zero_out <= 1'b0;

ALU_result_out <= 32'b0; Read_data_2_out <= 32'b0;

RegisterRd_out <= 5'b0;

end

else begin

RegWrite_out <= RegWrite_in; MemtoReg_out <= MemtoReg_in;

Branch_out <= Branch_in; MemRead_out <= MemRead_in;

MemWrite_out <= MemWrite_in; Jump_out <= Jump_in;

Branch_addr_out <= Branch_addr_in; ALU_zero_out <= ALU_zero_in;

ALU_result_out <= ALU_result_in; Read_data_2_out <= Read_data_2_in;

RegisterRd_out <= RegisterRd_in;

end

end

endmodule

// MEM/WB stage register

module MEM_WB_Stage_Reg (RegWrite_in, RegWrite_out,

MemtoReg_in, MemtoReg_out,

Data_memory_read_in, Data_memory_read_out,

ALU_result_in, ALU_result_out,

Write_Register_in, Write_Register_out, clk, reset);

// WB control signal

input RegWrite_in, MemtoReg_in;

output reg RegWrite_out, MemtoReg_out;

// data content

input [31:0] Data_memory_read_in, ALU_result_in;

output reg [31:0] Data_memory_read_out, ALU_result_out;

input [4:0] Write_Register_in;

output reg [4:0] Write_Register_out;

// general signal

input clk, reset;

always @(posedge clk or negedge reset) begin

if (!reset) begin

RegWrite_out <= 1'b0; MemtoReg_out <= 1'b0;

Data_memory_read_out <= 32'b0;  ALU_result_out <= 32'b0;

Write_Register_out <= 5'b0;

end

else begin

RegWrite_out <= RegWrite_in; MemtoReg_out <= MemtoReg_in;

Data_memory_read_out <= Data_memory_read_in; ALU_result_out <= ALU_result_in;

Write_Register_out <= Write_Register_in;

end

end

endmodule

// PC

module Program_Counter (clk, reset, PC_in, PC_out);

input clk, reset;

input [31:0] PC_in;

output reg [31:0] PC_out;

always @ (posedge clk or negedge reset) begin

if (!reset)

PC_out <= 0;

else

PC_out <= PC_in;

end

endmodule

// contains hard-code instructions

module Instruction_Memory (address, instruction, reset);

input reset;

input [31:0] address;

output [31:0] instruction;

reg [31:0] mem [7:0]; // 8 instructions

integer k;

// get instruction right away

assign instruction = mem[address[6:2]];

// Initial setup at reset posedge

always @(negedge reset) begin

for (k = 0; k < 8; k = k + 1) begin

mem[k] = 32'b0; // add $0 $0 $0

end

mem[0] = 32'b000000_00011_00100_00010_00000_100000; // add $2, $3, $4

mem[1] = 32'b000000_00011_00100_00001_00000_100010; // sub $1, $3, $4

mem[2] = 32'b100011_00110_00101_0000000000000000; // lw $5, 0($6)

mem[3] = 32'b000100_00011_00100_1111111111111100; // beq $3, $4, Label (-4)

end

endmodule

// 32-bit ALU for addition only

module ALU_add_only (input1, input2, add_out);

input [31:0] input1, input2;

output [31:0] add_out;

assign add_out=input1+input2;

endmodule

// N_bit_MUX for Usability

module N_bit_MUX (input0, input1, mux_out, control);

parameter N = 32;

input [N-1:0] input0, input1;

input control;

output [N-1:0] mux_out;

assign mux_out = control ? input1 : input0;

endmodule

// sync register file (write/read occupy half cycle each)

// write: on rising edge; data width 32 bit; address width 5 bit

// read: on falling edge; data width 32 bit; address width 5 bit

module Register_File (Read_Register_1, Read_Register_2,

Write_Register, Write_Data, Read_Data_1, Read_Data_2,

RegWrite, clk, reset);

input [4:0] Read_Register_1, Read_Register_2, Write_Register;

input [31:0] Write_Data;

input clk, reset, RegWrite;

output reg [31:0] Read_Data_1, Read_Data_2;

reg [31:0] mem [7:0];

integer k;

always @(posedge clk or negedge reset) begin

if (!reset) begin

for (k = 0; k < 8; k = k + 1) begin

mem[k] = 32'b0;

end

mem[3] = 32'b0011;

mem[4] = 32'b0011;

mem[6] = 32'h0000_0040;

end

else if (RegWrite)

mem[Write_Register] = Write_Data;

end

always @(negedge clk) begin

Read_Data_1 = mem[Read_Register_1];

Read_Data_2 = mem[Read_Register_2];

end

endmodule

// sign-extend the 16-bit input to the 32_bit output

module Sign_Extension (input_16, output_32);

input [15:0] input_16;

output [31:0] output_32;

assign output_32[15:0]  = input_16[15:0];

assign output_32[31:16] = input_16[15] ? 16'b1111_1111_1111_1111: 16'b0;

endmodule

// Control Path

module Control (OpCode, RegDst, Jump, Branch, MemRead, MemtoReg, ALUOp, MemWrite, ALUSrc, RegWrite);

input [5:0] OpCode;

output [1:0] ALUOp;

output RegDst, Jump, Branch, MemRead, MemtoReg, MemWrite, ALUSrc, RegWrite;

// 000000 : add, sub, and, or, slt

// 001000 : addi

// 100011 : lw

// 101011 : sw

// 000100 : beq

// 000010 : j

// 000000 (R-format)

assign RegDst=(~OpCode[5])&(~OpCode[4])&(~OpCode[3])&(~OpCode[2])&(~OpCode[1])&(~OpCode[0]);

// 000000 (R-format)

assign ALUOp[1]=(~OpCode[5])&(~OpCode[4])&(~OpCode[3])&(~OpCode[2])&(~OpCode[1])&(~OpCode[0]);

// 000100 (beq)

assign ALUOp[0]=(~OpCode[5])&(~OpCode[4])&(~OpCode[3])&(OpCode[2])&(~OpCode[1])&(~OpCode[0]);

// 100011 (lw), 101011 (sw)

assign ALUSrc=((OpCode[5])&(~OpCode[4])&(~OpCode[3])&(~OpCode[2])&(OpCode[1])&(OpCode[0]))  |

((OpCode[5])&(~OpCode[4])&(OpCode[3])&(~OpCode[2])&(OpCode[1])&(OpCode[0]));

// 000100 (beq)

assign Branch=(~OpCode[5])&(~OpCode[4])&(~OpCode[3])&(OpCode[2])&(~OpCode[1])&(~OpCode[0]);

// 100011 (lw)

assign MemRead=(OpCode[5])&(~OpCode[4])&(~OpCode[3])&(~OpCode[2])&(OpCode[1])&(OpCode[0]);

// 101011 (sw)

assign MemWrite=(OpCode[5])&(~OpCode[4])&(OpCode[3])&(~OpCode[2])&(OpCode[1])&(OpCode[0]);

// 100011 (lw)

assign MemtoReg=(OpCode[5])&(~OpCode[4])&(~OpCode[3])&(~OpCode[2])&(OpCode[1])&(OpCode[0]);

// 000000 (R-format), 001000 (addi), 001100, 100011 (lw)

assign RegWrite=((~OpCode[5])&(~OpCode[4])&(~OpCode[3])&(~OpCode[2])&(~OpCode[1])&(~OpCode[0]))|

((~OpCode[5])&(~OpCode[4])&(OpCode[3])&(~OpCode[2])&(~OpCode[1])&(~OpCode[0])) |

((OpCode[5])&(~OpCode[4])&(~OpCode[3])&(~OpCode[2])&(OpCode[1])&(OpCode[0]));

// 000010 (j)

assign Jump=(~OpCode[5])&(~OpCode[4])&(~OpCode[3])&(~OpCode[2])&(OpCode[1])&(~OpCode[0]);

endmodule

module ALU_Control (ALUOp, f_code, operation_code);

input [1:0] ALUOp;

input [5:0] f_code;

output [3:0] operation_code;

assign operation_code[3]=0;

// 0  1   | x x x x x x branch => subtract

// 1  0   | x x 0 0 1 0 => R-type subtract

// 1  0   | x x 1 0 1 0 => R-type slt

assign operation_code[2]=((~ALUOp[1])&(ALUOp[0])) |

((ALUOp[1])&(~ALUOp[0])&(~f_code[3])&(~f_code[2])&(f_code[1])&(~f_code[0])) |

((ALUOp[1])&(~ALUOp[0])&(f_code[3])&(~f_code[2])&(f_code[1])&(~f_code[0]));

// 0  0   | x x x x x x lw or sw => add

// 0  1   | x x x x x x branch => subtract

// 1  0   | x x 0 0 0 0 => R-type add

// 1  0   | x x 0 0 1 0 => R-type subtract

// 1  0   | x x 1 0 1 0 => R-type slt

assign operation_code[1]=((~ALUOp[1])&(~ALUOp[0])) |

((~ALUOp[1])&(ALUOp[0]))  |

((ALUOp[1])&(~ALUOp[0])&(~f_code[3])&(~f_code[2])&(~f_code[1])&(~f_code[0])) |

((ALUOp[1])&(~ALUOp[0])&(~f_code[3])&(~f_code[2])&(f_code[1])&(~f_code[0]))  |

((ALUOp[1])&(~ALUOp[0])&(f_code[3])&(~f_code[2])&(f_code[1])&(~f_code[0]));

// ALU OP | f_code field

// 1  0   | x x 0 1 0 1 => R-type Or

// 1  0   | x x 1 0 1 0 => R-type slt

assign operation_code[0]=((ALUOp[1])&(~ALUOp[0])&(~f_code[3])&(f_code[2])&(~f_code[1])&(f_code[0])) |

((ALUOp[1])&(~ALUOp[0])&(f_code[3])&(~f_code[2])&(f_code[1])&(~f_code[0]));

endmodule

// 32-bit ALU

module ALU (input1, input2, alu_out, zero, control);

// TODO : negative number handling

input [31:0] input1, input2;

input [3:0] control;

output reg [31:0] alu_out;

output reg zero;

always @ (control or input1 or input2) begin

case (control)

// and

4'b0000: begin alu_out<=input1&input2; zero<=0; end

// or

4'b0001: begin alu_out<=input1|input2; zero<=0; end

// add

4'b0010: begin alu_out<=input1+input2; zero<=0; end

// subtract

4'b0110: begin

if(input1 == input2)

zero <= 1;

else

zero <= 0;

alu_out <= input1 - input2;

end

// slt

4'b0111: begin

zero <= 0;

if(input1 - input2 >= 32'h8000_0000)

alu_out <= 32'b1;

else

alu_out <= 32'b0;

end

default: begin

zero <= 0;

alu_out <= input1;

end

endcase

end

endmodule

// Referred Previous Memory Project

module Data_Memory (MemAddr, Write_Data, Read_Data, clk, reset, MemRead, MemWrite);

input clk, reset;

input MemRead, MemWrite;

input [7:0] MemAddr;

input [31:0] Write_Data;

output reg [31:0] Read_Data;

reg [31:0] mem [63:0];

integer k;

always @(*) begin

if (!reset) begin

for (k = 0; k < 64; k = k + 1) begin

mem[k] = 32'b0;

end

mem[16] = 30;

end

else

if (MemRead && !MemWrite) begin

Read_Data = mem[MemAddr[7:2]];

end

else if (!MemRead && MemWrite) begin

mem[MemAddr[7:2]] = Write_Data;

end

else begin

Read_Data = 32'bx;

end

end

endmodule

2. 실행 결과

위 코드는 상단 조건에 따라 다음과 같은 작업을 반복한다.

Beq에 의해 다시 Label로 되돌아가기 때문에, 같은 과정이 반복된다.

| PC Value | Fetch | Decode | Execute | Memory | Write Back |
| --- | --- | --- | --- | --- | --- |
| 00000 | Add |  |  |  |  |
| 00100 | Sub | Add |  |  |  |
| 01000 | lw | Sub | Add |  |  |
| 01100 | beq | lw | Sub | Add |  |
| 10000 | NOP | beq | lw | Sub | Add |
| 10100 | NOP | NOP | beq | lw | Sub |
| 11000 | NOP | NOP | NOP | beq | lw |
| 00000 | Add (반복) | NOP | NOP | NOP | beq |
| 00100 | Sub | Add (반복) | NOP | NOP | NOP |

각 매개변수들에 대한 설명은 다음과 같다.

- Result : ALU 계산 결과 (후에 FPGA LED 값이 된다.)

- PC_Now : 현 시점에서 Instruction Fetch에서 사용되는 PC를 나타냄

- Instruction_Now : 현 시점에서 Instruction Decode에서 사용되는 Instruction을 나타냄

- Rs_Now & Rt_Now : Instruction_Now를 통해 Register_File에서 가져온 Rs,Rt 값이다. Falling Edge에서 최신화됨

- ALU_Input_1, ALU_Input_2 : Execute 단계에서 ALU의 input이 되는 두 피연산자

- Load_data : Memory 단계에서 읽어온 데이터 (현재는 lw에서만 해당)

- Beq_Address : Execute 단계에서 계산되는 branch address

- Immi_Shifted : Execute 단계에서 계산되는 32비트 shift된 값 (이것이 PC와 합쳐져 Beq_Address가 된다.)

- PC_4 : Execute 단계에서의 PC, 큰 의미는 없다. 편의를 위해 추가

- Debug_flag_2 : 편의상 이것저것을 확인하기 위한 flag, 큰 의미는 없다.

- Debug_Write_Data, Write_Register : Decode 단계의 Register_File에 Write되는 Register Address와 Data

- Debug_RegWrite : Write Back 단계에서 결정되는 RegWrite값

디버깅 Flag들을 포함하여 실행시킨 각 단계에서의 Waveform은 다음과 같다.

각 Pipeline 단계별로 결과를 구조화해보면 다음과 같다.

| PC_Now | Fetch | Decode | Execute | Memory | Write Back |
| --- | --- | --- | --- | --- | --- |
| 00000 | Instruction_Now = Add |  |  |  |  |
| 00100 | Instruction_Now = Sub | Rs=$3 / Rt=$4 |  |  |  |
| 01000 | Instruction_Now = lw | Rs=$3 / Rt=$4 | Result=$3+$4=6 |  |  |
| 01100 | Instruction_Now = beq | Rs=$6 / Rt=$5 | Result=$3-$4=0 | Add이므로 Nop |  |
| 10000 | NOP | Rs=$3 / Rt=$4 | Result=$6+0=0x40 | Sub이므로 Nop | $2에 결과 6저장요청 |
| 10100 | NOP | NOP | Result=$3-$4 | Mem[0x40] Read | $1에 결과 0저장요청 |
| 11000 | NOP | NOP | NOP | Branch 주소 계산 (PCSrc 변경) | $5에 결과 30저장요청 |
| 00000 | Branch & Add (반복) | NOP | NOP | NOP | Beq이므로 Nop |
| 00100 | 계속 반복 | Rs=$3 / Rt=$4 | NOP | NOP | NOP |

3. 코드 설명

필요하다고 판단되는 Unit에 대한 코드 설명을 추가한다.

- Program Counter

// PC

module Program_Counter (clk, reset, PC_in, PC_out);

input clk, reset;

input [31:0] PC_in;

output reg [31:0] PC_out;

always @ (posedge clk or negedge reset) begin

if (!reset)

PC_out <= 0;

else

PC_out <= PC_in;

end

endmodule

다음 PC로 업데이트, 혹은 reset에 의해 업데이트를 중지하는 기능을 갖는다.

다음 PC는 PC + 4 혹은, Jump/Branch Address가 될 수 있다.

- Instruction Memory

// contains hard-code instructions

module Instruction_Memory (address, instruction, reset);

input reset;

input [31:0] address;

output [31:0] instruction;

reg [31:0] mem [7:0]; // 8 instructions

integer k;

// get instruction right away

assign instruction = mem[address[6:2]];

// Initial setup at reset posedge

always @(negedge reset) begin

for (k = 0; k < 8; k = k + 1) begin

mem[k] = 32'b0; // add $0 $0 $0

end

mem[0] = 32'b000000_00011_00100_00010_00000_100000; // add $2, $3, $4

mem[1] = 32'b000000_00011_00100_00001_00000_100010; // sub $1, $3, $4

mem[2] = 32'b100011_00110_00101_0000000000000000; // lw $5, 0($6)

mem[3] = 32'b000100_00011_00100_1111111111111100; // beq $3, $4, Label (-4)

end

endmodule

PC에 따라 새로운 Instruction을 불러온다.

이때, PC는 word단위로 update되기에, 하위 2비트는 사용하지 않게 함으로 사용 메모리를 줄일 수 있다.

Instruction들을 Hard Coding 되어 있다.

- IF_ID_Stage

// IF/ID stage register

module IF_ID_Stage_Reg (clk, reset, PC_plus4_in, PC_plus4_out,

instruction_in, instruction_out);

input clk, reset;

input [31:0] PC_plus4_in, instruction_in;

output reg [31:0] PC_plus4_out, instruction_out;

always @(posedge clk or negedge reset) begin

if (!reset) begin

PC_plus4_out <= 32'b0;

instruction_out <= 32'b0;

end

else begin

PC_plus4_out <= PC_plus4_in;

instruction_out <= instruction_in;

end

end

endmodule

Fetch => Decode로 넘어가면서, 필요한 데이터들을 넘겨주는 역할을 한다.

Reset시, 데이터는 모두 초기화된다.

Decode => Execute, Execute => Memory, Memory => Write Back 모두 비슷한 구조를 갖기에 여기에서의 설명과 동일하다.

위 Unit들을 통해 Instruction Fetch는 다음과 같이 구성된다.

//////////////////////////////////

//// Instruction Fetch stage /////

//////////////////////////////////

N_bit_MUX #(32) pc_mux (.input0(PC_plus4), .input1(MEM_Branch_addr),

.mux_out(PC_in), .control(PCSrc));

Program_Counter program_counter (.clk(clk), .reset(reset),

.PC_in(PC_in), .PC_out(PC_out));

Instruction_Memory instruction_memory (.address(PC_out),

.instruction(IF_instruction), .reset(reset));

ALU_add_only pc_add_4 (.input1(PC_out), .input2(32'b0100), .add_out(PC_plus4));

IF_ID_Stage_Reg IF_ID_Stage_Unit (.clk(clk), .reset(reset),

.PC_plus4_in(PC_plus4), .PC_plus4_out(ID_PC_plus4),

.instruction_in(IF_instruction), .instruction_out(ID_instruction));

다른 Pipeline 단계들도 비슷하게 구성되며, 이는 주어진 Diagram에 따라 배치 및 연결된다.

- Register File

module Register_File (Read_Register_1, Read_Register_2,

Write_Register, Write_Data, Read_Data_1, Read_Data_2,

RegWrite, clk, reset);

input [4:0] Read_Register_1, Read_Register_2, Write_Register;

input [31:0] Write_Data;

input clk, reset, RegWrite;

output reg [31:0] Read_Data_1, Read_Data_2;

reg [31:0] mem [7:0];

integer k;

always @(posedge clk or negedge reset) begin

if (!reset) begin

for (k = 0; k < 8; k = k + 1) begin

mem[k] = 32'b0;

end

mem[3] = 32'b0011;

mem[4] = 32'b0011;

mem[6] = 32'h0000_0040;

end

else if (RegWrite)

mem[Write_Register] = Write_Data;

end

always @(negedge clk) begin

Read_Data_1 = mem[Read_Register_1];

Read_Data_2 = mem[Read_Register_2];

end

endmodule

레지스터의 초기화 및 Read/Write를 담당하는 Unit이다.

Posedge와 Negedge로 나뉘어 동작하며, Posedge시는 Register Write를, Negedge시는 Register Read를 수행한다.

이를 통해 Write로 Register를 업데이트함과 동시에 Read하여 사용할 수 있게 된다.

- Control

// Control Path

module Control (OpCode, RegDst, Jump, Branch, MemRead, MemtoReg, ALUOp, MemWrite, ALUSrc, RegWrite);

input [5:0] OpCode;

output [1:0] ALUOp;

output RegDst, Jump, Branch, MemRead, MemtoReg, MemWrite, ALUSrc, RegWrite;

// 000000 : add, sub, and, or, slt

// 001000 : addi

// 100011 : lw

// 101011 : sw

// 000100 : beq

// 000010 : j

// 000000 (R-format)

assign RegDst=(~OpCode[5])&(~OpCode[4])&(~OpCode[3])&(~OpCode[2])&(~OpCode[1])&(~OpCode[0]);

// 000000 (R-format)

assign ALUOp[1]=(~OpCode[5])&(~OpCode[4])&(~OpCode[3])&(~OpCode[2])&(~OpCode[1])&(~OpCode[0]);

// 000100 (beq)

assign ALUOp[0]=(~OpCode[5])&(~OpCode[4])&(~OpCode[3])&(OpCode[2])&(~OpCode[1])&(~OpCode[0]);

// 100011 (lw), 101011 (sw)

assign ALUSrc=((OpCode[5])&(~OpCode[4])&(~OpCode[3])&(~OpCode[2])&(OpCode[1])&(OpCode[0]))  |

((OpCode[5])&(~OpCode[4])&(OpCode[3])&(~OpCode[2])&(OpCode[1])&(OpCode[0]));

// 000100 (beq)

assign Branch=(~OpCode[5])&(~OpCode[4])&(~OpCode[3])&(OpCode[2])&(~OpCode[1])&(~OpCode[0]);

// 100011 (lw)

assign MemRead=(OpCode[5])&(~OpCode[4])&(~OpCode[3])&(~OpCode[2])&(OpCode[1])&(OpCode[0]);

// 101011 (sw)

assign MemWrite=(OpCode[5])&(~OpCode[4])&(OpCode[3])&(~OpCode[2])&(OpCode[1])&(OpCode[0]);

// 100011 (lw)

assign MemtoReg=(OpCode[5])&(~OpCode[4])&(~OpCode[3])&(~OpCode[2])&(OpCode[1])&(OpCode[0]);

// 000000 (R-format), 001000 (addi), 001100, 100011 (lw)

assign RegWrite=((~OpCode[5])&(~OpCode[4])&(~OpCode[3])&(~OpCode[2])&(~OpCode[1])&(~OpCode[0]))|

((~OpCode[5])&(~OpCode[4])&(OpCode[3])&(~OpCode[2])&(~OpCode[1])&(~OpCode[0])) |

((OpCode[5])&(~OpCode[4])&(~OpCode[3])&(~OpCode[2])&(OpCode[1])&(OpCode[0]));

// 000010 (j)

assign Jump=(~OpCode[5])&(~OpCode[4])&(~OpCode[3])&(~OpCode[2])&(OpCode[1])&(~OpCode[0]);

endmodule

Instruction의 Opcode를 읽고, 해당하는 control signal을 생성한다.

각 경우에 대해서는 다음 표와 같이 매칭된다.

다음으로 ID_EX_Stage를 거쳐 Execute 단계에 진입한다.

N_bit_MUX #(32) alu_input_mux (.input0(EX_Read_data_2), .input1(EX_sign_extended_immi),

.mux_out(ALU_input_B), .control(EX_ALUSrc));

ALU에 진입하기 전, 두번째 피연산자를 선택하기 위해 MUX가 추가된 모습을 볼 수 있다.

이는 R-type과 I-type이 차이를 가질 것이다.

- ALU_Control

module ALU_Control (ALUOp, f_code, operation_code);

input [1:0] ALUOp;

input [5:0] f_code;

output [3:0] operation_code;

assign operation_code[3]=0;

// 0  1   | x x x x x x branch => subtract

// 1  0   | x x 0 0 1 0 => R-type subtract

// 1  0   | x x 1 0 1 0 => R-type slt

assign operation_code[2]=((~ALUOp[1])&(ALUOp[0])) |

((ALUOp[1])&(~ALUOp[0])&(~f_code[3])&(~f_code[2])&(f_code[1])&(~f_code[0])) |

((ALUOp[1])&(~ALUOp[0])&(f_code[3])&(~f_code[2])&(f_code[1])&(~f_code[0]));

// 0  0   | x x x x x x lw or sw => add

// 0  1   | x x x x x x branch => subtract

// 1  0   | x x 0 0 0 0 => R-type add

// 1  0   | x x 0 0 1 0 => R-type subtract

// 1  0   | x x 1 0 1 0 => R-type slt

assign operation_code[1]=((~ALUOp[1])&(~ALUOp[0])) |

((~ALUOp[1])&(ALUOp[0]))  |

((ALUOp[1])&(~ALUOp[0])&(~f_code[3])&(~f_code[2])&(~f_code[1])&(~f_code[0])) |

((ALUOp[1])&(~ALUOp[0])&(~f_code[3])&(~f_code[2])&(f_code[1])&(~f_code[0]))  |

((ALUOp[1])&(~ALUOp[0])&(f_code[3])&(~f_code[2])&(f_code[1])&(~f_code[0]));

// ALU OP | f_code field

// 1  0   | x x 0 1 0 1 => R-type Or

// 1  0   | x x 1 0 1 0 => R-type slt

assign operation_code[0]=((ALUOp[1])&(~ALUOp[0])&(~f_code[3])&(f_code[2])&(~f_code[1])&(f_code[0])) |

((ALUOp[1])&(~ALUOp[0])&(f_code[3])&(~f_code[2])&(f_code[1])&(~f_code[0]));

endmodule

Opcode로 구분할 수 없는 연산자들을 다시 한 번 구분하기 위해 사용된다.

복잡해 보이지만 아래 표를 그대로 구현한 것일 뿐이다.

- ALU

// 32-bit ALU

module ALU (input1, input2, alu_out, zero, control);

// TODO : negative number handling

input [31:0] input1, input2;

input [3:0] control;

output reg [31:0] alu_out;

output reg zero;

always @ (control or input1 or input2) begin

case (control)

// and

4'b0000: begin alu_out<=input1&input2; zero<=0; end

// or

4'b0001: begin alu_out<=input1|input2; zero<=0; end

// add

4'b0010: begin alu_out<=input1+input2; zero<=0; end

// subtract

4'b0110: begin

if(input1 == input2)

zero <= 1;

else

zero <= 0;

alu_out <= input1 - input2;

end

// slt

4'b0111: begin

zero <= 0;

if(input1 - input2 >= 32'h8000_0000)

alu_out <= 32'b1;

else

alu_out <= 32'b0;

end

default: begin

zero <= 0;

alu_out <= input1;

end

endcase

end

endmodule

ALU_Control의 output을 받아 해당하는 연산을 수행한다.

And, Or, Add, Sub, Slt 를 구현하였다.

- Branch 주소 계산

assign Shifted_immi = { EX_sign_extended_immi[29:0], 2'b00 };

ALU_add_only alu_add_only_unit (.input1(EX_PC_plus4), .input2(Shifted_immi), .add_out(Branch_addr));

I-type의 immediate 부분만을 추려 sign extension 한 뒤, shift 2 bit를 수행한다.

이후, 해당 Instruction Fetch 당시의 PC + 4와 Add 하여 Branch Address를 계산한다.

다음으로 Memory 단계를 살펴본다.

- Data Memory

// Referred Previous Memory Project

module Data_Memory (MemAddr, Write_Data, Read_Data, clk, reset, MemRead, MemWrite);

input clk, reset;

input MemRead, MemWrite;

input [7:0] MemAddr;

input [31:0] Write_Data;

output reg [31:0] Read_Data;

reg [31:0] mem [63:0];

integer k;

always @(*) begin

if (!reset) begin

for (k = 0; k < 64; k = k + 1) begin

mem[k] = 32'b0;

end

mem[16] = 30;

end

else

if (MemRead && !MemWrite) begin

Read_Data = mem[MemAddr[7:2]];

end

else if (!MemRead && MemWrite) begin

mem[MemAddr[7:2]] = Write_Data;

end

else begin

Read_Data = 32'bx;

end

end

endmodule

내부 Memory에 저장되어 있는 Data의 Read/Write를 수행하며, 이전 과제와 큰 차이는 없다.

- PCSrc를 위한 and

and (PCSrc, MEM_Branch, MEM_ALU_zero);

Diagram과 같이 Control로부터의 Branch와 ALU의 뺄셈 결과인 zero를 and 하여 PCSrc를 업데이트한다.

다음으로 Write Back 단계를 살핀다.

N_bit_MUX #(32) write_data_mux (.input0(WB_ALU_result), .input1(WB_Data_memory_read),

.mux_out(WB_Write_Data), .control(WB_MemtoReg));

Basic Pipeline의 Write Back 단계에서는 Mux만을 갖는다.

MemtoReg 값에 따라 다시 Register_File로 저장할 데이터를 전송할 때, ALU의 결과를 저장할 지, 혹은 Data로 부터 가져온 데이터를 저장할 지 선택하게 된다.

추가적으로 언급하지는 않았지만 자주 사용되는 Unit들은 다음과 같다.

- N_Bit_MUX : 32비트, 5비트 등 다양한 output을 갖는 mux가 필요한데, 이를 따로 만들지 않고, parameter를 사용한 N_Bit_MUX를 구현하였다.

- ALU_Add_only : 오로지 덧셈만을 수행하는 ALU이다.

- Sign_Extension : 최상위 비트에 따라 Sign_Extension을 수행한다.

다음으로 FPGA 구동을 위한 Startup 코드를 보자.

- Startup

제공되는 예제와 차이나는 부분만을 첨부하였다.

assign clk_slow = clk_operating;

basic_pipeline my_pipeline(.clk(clk_slow), .reset(reset), .result(ALU_Result));

//Counter

always @(posedge clk_50MHz or negedge reset) begin

if(!reset) begin

counter<=0;

clk_operating <= 1'b0;

end

else begin

counter<=counter+1;

clk_operating <= ((counter == 25'b1_1111_1111_1111_1111_1111_1111) ? ~clk_operating: clk_operating );

end

end

// // Test Led

// always @(posedge counter[24] or negedge reset) begin

//  if (!reset) begin

//      led <= 16'b0;

//  end

//  else begin

//      // 7 9 10 ... 23

//      led<= ~test_led;

//      test_led <= ~test_led;

//  end

// end

//Led

//always @(posedge counter[24] or negedge reset) begin

always @(posedge clk_50MHz or negedge reset) begin

if (!reset) begin

led <= 16'b0;

end

else begin

// 7 9 10 ... 23

led<= ALU_Result[15:0];

end

end

느린 clock을 사용하기 위해, 2^25비트마다 반복되는 clk_slow를 새로 선언하고, 해당 값을 pipeline에 연결한 모습을 볼 수 있다.

더불어, led에는 ALU결과의 하위 16비트를 매칭해 두었다.

LED의 결과, 즉 ALU의 결과는 다음 값들을 반복한다.

상단의 표 중에서 Execute단만을 가져왔으며, 결과는 2진수로 LED 위에 표현될 것이다.

| PC_Now | Execute |
| --- | --- |
| 00000 | All Zero |
| 00100 | All Zero |
| 01000 | Result=$3+$4=6 |
| 01100 | Result=$3-$4=0 |
| 10000 | Result=$6+0=0x40 |
| 10100 | Result=$3-$4=0 |
| 11000 | All Zero |
| 00000 | All Zero |
| 00100 | All Zero |

마지막으로 첨부하는 TB 파일들은 다음과 같다.

tb_basic_pipeline : clk, reset, result만을 output으로 갖는 test bench

tb_basic_pipeline_simple_ver : 각종 디버깅 output들을 모두 output으로 갖는 testbench

tb_data_memory : Data Memory를 위한 testbench

tb_control : Control을 위한 testbench

tb_register_file : Register File을 위한 testbench

tb_instruction_memory : instruction memory를 위한 testbench