| 컴퓨터구조 결 과 보 고 서 (Result report) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Major | Student ID | Grade | Name | Experiment date |
| 융합전자공학부 | 2015036580 | 3 | 김수영 | 2021-06-24 |
| Title | Advanced Pipeline 과제 |  |  |  |

1. 전체 코드 첨부

(디버깅을 위한 output은 주석 처리되어 있음)

`timescale 1ns / 1ps

//////////////////////////////////////////////////////////////////////////////////

// Company:

// Engineer:

//

// Create Date:    12:48:03 06/22/2021

// Design Name:

// Module Name:    advanced_pipeline

// Project Name:

// Target Devices:

// Tool versions:

// Description:

//

// Dependencies:

//

// Revision:

// Revision 0.01 - File Created

// Additional Comments:

//

//////////////////////////////////////////////////////////////////////////////////

module advanced_pipeline (clk, reset, result, PC_now,

d_Branch, d_Jump, d_RS_RT_Equal, d_Branch_Taken);

// PC_now, Instruction_now,

// Rs_now, Rt_now, ALU_input_1, ALU_input_2, immi_Shifted, PC_4,

// load_data, Beq_address, J_address, Write_Register,

// debug_flag_2, Debug_RegWrite, Debug_Write_Data,

// Debug_PCWrite, Debug_IF_ID_Write, Debug_IF_Flush,

// Debug_ForWardA, Debug_ForWardB);

input clk, reset;

output [31:0] result, PC_now; // ALU result

input d_Branch, d_Jump, d_RS_RT_Equal, d_Branch_Taken;

// output [31:0] load_data; // Data_memory_read

// output [31:0] PC_now; // current PC value Debugging,

// output [31:0] Instruction_now; // Fetch stage instruction

// output [31:0] Rs_now, Rt_now; // output from Register File

// output [31:0] ALU_input_1, ALU_input_2; // ALU inputs

// output [31:0] PC_4, Beq_address, immi_Shifted; // for Branch Address Debugging

// output [31:0] J_address; // Jump Address

// output [4:0] Write_Register; // Rd value

// output [31:0] debug_flag_2, Debug_Write_Data; // ALU_result in MEM stage, Write_Data for Register File

// output Debug_RegWrite; // RegWrite Op Flag

// output Debug_PCWrite, Debug_IF_ID_Write, Debug_IF_Flush;

// output [1:0] Debug_ForWardA, Debug_ForWardB;

// IF stage

wire [31:0] PC_in;

wire [31:0] PC_out;

wire [31:0] PC_plus4;

wire [31:0] IF_instruction;

wire [31:0] Branch_MUX_output;

wire [31:0] Jump_MUX_output;

wire PCWrite;

wire IF_ID_Write;

wire Branch_taken;

// ID stage

wire [31:0] ID_PC_plus4;

wire [31:0] ID_instruction;

wire [31:0] Read_data_1, Read_data_2;

wire [31:0] sign_extended_immi;

wire [31:0] Jump_Address;

wire [31:0] ID_Shifted_immi;

wire [31:0] ID_Branch_addr;

wire ALUSrc, RegWrite;

wire RegDst, Jump, Branch;

wire MemRead, MemtoReg, MemWrite;

wire RegDst_t, Jump_t, Branch_t;

wire MemRead_t, MemtoReg_t, MemWrite_t;

wire IF_Flush, ID_Flush, EX_Flush;

wire Rs_Rt_equal;

wire [1:0] ALUOp;

wire Hazard, Control_Reset;

// EX stage

wire [31:0] EX_PC_plus4;

wire [31:0] EX_instruction;

wire [31:0] EX_Read_data_1, EX_Read_data_2;

wire [31:0] EX_sign_extended_immi;

wire EX_ALUSrc, EX_RegWrite;

wire EX_RegDst, EX_Jump, EX_Branch;

wire EX_MemRead, EX_MemtoReg, EX_MemWrite;

wire [1:0] EX_ALUOp;

wire [4:0] EX_Write_Register;

wire [3:0] operation_code;

wire [31:0] Branch_addr;

wire [31:0] Shifted_immi;

wire [31:0] ALU_input_B;

wire [31:0] ALU_result;

wire ALU_zero;

wire [1:0] ForwardA, ForwardB;

wire [31:0] muxA_out, muxB_out;

// MEM stage

wire [31:0] MEM_ALU_result, MEM_Read_data_2;

wire [4:0] MEM_Write_Register;

wire [31:0] Data_memory_read;

wire [31:0] MEM_Branch_addr;

wire MEM_RegWrite, MEM_MemtoReg, MEM_Branch;

wire MEM_MemRead, MEM_MemWrite, MEM_Jump;

wire MEM_ALU_zero;

wire PCSrc;

// WB stage

wire [31:0] WB_Data_memory_read, WB_ALU_result;

wire [4:0] WB_Write_Register;

wire [31:0] WB_Write_Data;

wire WB_RegWrite, WB_MemtoReg;

//////////////////////////////////

//// Instruction Fetch stage /////

//////////////////////////////////

Program_Counter program_counter (.clk(clk), .reset(reset),

.PC_in(PC_in), .PC_out(PC_out), .PCWrite(PCWrite));

Instruction_Memory instruction_memory (.address(PC_out),

.instruction(IF_instruction), .reset(reset));

ALU_add_only pc_add_4 (.input1(PC_out), .input2(32'b0100), .add_out(PC_plus4));

// assign Debug_PCWrite = PCWrite;

// assign Debug_IF_Flush = IF_Flush;

assign Branch_taken = Branch & Rs_Rt_equal;

// and (Branch_taken, Branch, Rs_Rt_equal);

N_bit_MUX #(32) branch_mux (.input0(PC_plus4), .input1(ID_Branch_addr),

.mux_out(Branch_MUX_output), .control(Branch_taken));

N_bit_MUX #(32) jump_mux (.input0(Branch_MUX_output), .input1(Jump_Address),

.mux_out(PC_in), .control(Jump));

assign d_Jump = Jump;

assign d_Branch = Branch;

assign d_RS_RT_Equal = Rs_Rt_equal;

assign d_Branch_Taken = Branch_taken;

IF_ID_Stage_Reg IF_ID_Stage_Unit (.clk(clk), .reset(reset),

.IF_ID_Write(IF_ID_Write), .IF_Flush(IF_Flush),

.PC_plus4_in(PC_plus4), .PC_plus4_out(ID_PC_plus4),

.instruction_in(IF_instruction), .instruction_out(ID_instruction));

assign PC_now = PC_out;

// assign Instruction_now = IF_instruction;

// assign Debug_IF_ID_Write = IF_ID_Write;

//////////////////////////////////

//// Instruction Decode stage ////

//////////////////////////////////

// assign Debug_RegWrite = WB_RegWrite;

// assign Debug_Write_Data = WB_Write_Data;

Register_File regfile_Unit (.clk(clk), .reset(reset),

.Read_Register_1(ID_instruction[25:21]),

.Read_Register_2(ID_instruction[20:16]),

.Write_Register(WB_Write_Register), .Write_Data(WB_Write_Data),

.Read_Data_1(Read_data_1), .Read_Data_2(Read_data_2),

.RegWrite(WB_RegWrite));

assign Rs_Rt_equal = (Read_data_1 == Read_data_2)?1:0;

assign Jump_Address = {ID_PC_plus4[31:28], ID_instruction[25:0], 2'b00};

Sign_Extension immi_sign_extension (.input_16(ID_instruction[15:0]), .output_32(sign_extended_immi));

assign ID_Shifted_immi = { sign_extended_immi[29:0], 2'b00 };

ALU_add_only alu_add_only_unit (.input1(ID_PC_plus4),

.input2(ID_Shifted_immi), .add_out(ID_Branch_addr));

// assign immi_Shifted = ID_Shifted_immi;

// assign Beq_address = ID_Branch_addr;

// assign J_address = Jump_Address;

Hazard_Detection_Unit hazard_detection_unit (.ID_EX_RegisterRt(EX_instruction[20:16]), .ID_EX_MemRead(EX_MemRead),

.IF_ID_RegisterRs(ID_instruction[25:21]), .IF_ID_RegisterRt(ID_instruction[20:16]),

.PCWrite(PCWrite), .IF_ID_Write(IF_ID_Write), .Hazard(Hazard));

Control control_unit (.OpCode(ID_instruction[31:26]),

.RegDst(RegDst), .Jump(Jump), .Branch(Branch),

.MemRead(MemRead), .MemtoReg(MemtoReg), .ALUOp(ALUOp),

.MemWrite(MemWrite), .ALUSrc(ALUSrc), .RegWrite(RegWrite),

.IF_Flush(IF_Flush), .ID_Flush(ID_Flush), .EX_Flush(EX_Flush));

or (Control_Reset, ID_Flush, Hazard);

Reset_Control reset_control_unit(

.RegDst_in(RegDst), .ALUSrc_in(ALUSrc), .Branch_in(Branch),

.MemRead_in(MemRead), .MemtoReg_in(MemtoReg),

.MemWrite_in(MemWrite), .RegWrite_in(RegWrite),

.RegDst_out(RegDst_t), .ALUSrc_out(ALUSrc_t), .Branch_out(Branch_t),

.MemRead_out(MemRead_t), .MemtoReg_out(MemtoReg_t),

.MemWrite_out(MemWrite_t), .RegWrite_out(RegWrite_t),

.Control_Reset(Control_Reset));

ID_EX_Stage_Reg ID_EX_Stage_Unit (.clk(clk), .reset(reset),

.RegWrite_in(RegWrite_t), .RegWrite_out(EX_RegWrite),

.MemtoReg_in(MemtoReg_t), .MemtoReg_out(EX_MemtoReg),

.Branch_in(Branch_t), .Branch_out(EX_Branch),

.MemRead_in(MemRead_t), .MemRead_out(EX_MemRead),

.MemWrite_in(MemWrite_t), .MemWrite_out(EX_MemWrite),

.Jump_in(Jump), .Jump_out(EX_Jump),

.RegDst_in(RegDst_t), .RegDst_out(EX_RegDst),

.ALUSrc_in(ALUSrc_t), .ALUSrc_out(EX_ALUSrc),

.ALUOp_in(ALUOp), .ALUOp_out(EX_ALUOp),

.PC_plus4_in(ID_PC_plus4), .PC_plus4_out(EX_PC_plus4),

.read_data_1_in(Read_data_1), .read_data_1_out(EX_Read_data_1),

.read_data_2_in(Read_data_2), .read_data_2_out(EX_Read_data_2),

.sign_extended_immi_in(sign_extended_immi), .sign_extended_immi_out(EX_sign_extended_immi),

.instruction_in(ID_instruction), .instruction_out(EX_instruction));

// assign Rs_now = Read_data_1;

// assign Rt_now = Read_data_2;

//////////////////////////////////

///////   Execute  stage  ////////

//////////////////////////////////

N_bit_MUX #(5) write_reg_mux (.input0(EX_instruction[20:16]), .input1(EX_instruction[15:11]),

.mux_out(EX_Write_Register), .control(EX_RegDst));

ALU_Control alu_control_unit (.ALUOp(EX_ALUOp), .f_code(EX_instruction[5:0]), .operation_code(operation_code));

Forwarding_Unit forwarding_unit (.EX_RegisterRs(EX_instruction[25:21]), .EX_RegisterRt(EX_instruction[20:16]),

.MEM_RegisterRd(MEM_Write_Register), .WB_RegisterRd(WB_Write_Register),

.EX_MEM_RegWrite(MEM_RegWrite), .MEM_WB_RegWrite(WB_RegWrite),

.ForwardA(ForwardA), .ForwardB(ForwardB));

Mux_32bit_3to1 ALU_MuxA (.in00(EX_Read_data_1), .in01(WB_Write_Data), .in10(MEM_ALU_result),

.mux_out(muxA_out), .control(ForwardA));

Mux_32bit_3to1 ALU_MuxB (.in00(EX_Read_data_2), .in01(WB_Write_Data), .in10(MEM_ALU_result),

.mux_out(muxB_out), .control(ForwardB));

// assign Debug_ForWardA = ForwardA;

// assign Debug_ForWardB = ForwardB;

N_bit_MUX #(32) alu_input_mux (.input0(muxB_out), .input1(EX_sign_extended_immi),

.mux_out(ALU_input_B), .control(EX_ALUSrc));

ALU alu_unit (.input1(muxA_out), .input2(ALU_input_B),

.alu_out(ALU_result), .zero(ALU_zero), .control(operation_code));

// assign Shifted_immi = { EX_sign_extended_immi[29:0], 2'b00 };

// ALU_add_only alu_add_only_unit (.input1(EX_PC_plus4),

// .input2(Shifted_immi), .add_out(Branch_addr));

EX_MEM_Stage_Reg EX_MEM_Stage_Unit ( .clk(clk), .reset(reset),

.RegWrite_in(EX_RegWrite), .RegWrite_out(MEM_RegWrite),

.MemtoReg_in(EX_MemtoReg), .MemtoReg_out(MEM_MemtoReg),

.Branch_in(EX_Branch), .Branch_out(MEM_Branch),

.MemRead_in(EX_MemRead), .MemRead_out(MEM_MemRead),

.MemWrite_in(EX_MemWrite),.MemWrite_out(MEM_MemWrite),

.Jump_in(EX_Jump), .Jump_out(MEM_Jump),

// .Branch_addr_in(Branch_addr), .Branch_addr_out(MEM_Branch_addr),

.ALU_zero_in(ALU_zero), .ALU_zero_out(MEM_ALU_zero),

.ALU_result_in(ALU_result), .ALU_result_out(MEM_ALU_result),

.Read_data_2_in(EX_Read_data_2), .Read_data_2_out(MEM_Read_data_2),

.RegisterRd_in(EX_Write_Register), .RegisterRd_out(MEM_Write_Register));

// assign PC_4 = EX_PC_plus4;

// assign ALU_input_1 = muxA_out;

// assign ALU_input_2 = ALU_input_B;

assign result = { 27'b0, ALU_result[4:0]};

//////////////////////////////////

///////   Memory  stage  /////////

//////////////////////////////////

Data_Memory data_memory_unit (.clk(clk), .reset(reset), .MemAddr(MEM_ALU_result[7:0]),

.Write_Data(MEM_Read_data_2), .Read_Data(Data_memory_read),

.MemRead(MEM_MemRead), .MemWrite(MEM_MemWrite));

and (PCSrc, MEM_Branch, MEM_ALU_zero);

// assign debug_flag_2 = MEM_ALU_result;

// assign load_data = Data_memory_read;

MEM_WB_Stage_Reg MEM_WB_Stage_Unit (.clk(clk), .reset(reset),

.RegWrite_in(MEM_RegWrite), .RegWrite_out(WB_RegWrite),

.MemtoReg_in(MEM_MemtoReg), .MemtoReg_out(WB_MemtoReg),

.Data_memory_read_in(Data_memory_read), .Data_memory_read_out(WB_Data_memory_read),

.ALU_result_in(MEM_ALU_result), .ALU_result_out(WB_ALU_result),

.Write_Register_in(MEM_Write_Register), .Write_Register_out(WB_Write_Register));

//////////////////////////////////

///   Write Back stage      //////

//////////////////////////////////

N_bit_MUX #(32) write_data_mux (.input0(WB_ALU_result), .input1(WB_Data_memory_read),

.mux_out(WB_Write_Data), .control(WB_MemtoReg));

// assign Write_Register = WB_Write_Register;

endmodule

// IF/ID stage register

module IF_ID_Stage_Reg (clk, reset,

PC_plus4_in, PC_plus4_out,

instruction_in, instruction_out,

IF_ID_Write, IF_Flush);

input clk, reset;

input [31:0] PC_plus4_in, instruction_in;

output reg [31:0] PC_plus4_out, instruction_out;

// Hazard

input IF_Flush, IF_ID_Write;

always @(posedge clk or posedge reset) begin

if (reset) begin

PC_plus4_out <= 32'b0;

instruction_out <= 32'b0;

end

else if (IF_Flush) begin

// TODO : Check condition

// PC_plus4_out <= 32'b0;

// PC_plus4_out <= PC_plus4_in;

instruction_out <= 32'b0;

end

else if (IF_ID_Write) begin

PC_plus4_out <= PC_plus4_in;

instruction_out <= instruction_in;

end

end

endmodule

// ID/EX stage register

module ID_EX_Stage_Reg (clk, reset, RegWrite_in, RegWrite_out, MemtoReg_in, MemtoReg_out,

Branch_in, Branch_out, MemRead_in, MemRead_out, MemWrite_in, MemWrite_out,

Jump_in, Jump_out, RegDst_in, RegDst_out, ALUSrc_in, ALUSrc_out, ALUOp_in, ALUOp_out,

PC_plus4_in, PC_plus4_out, read_data_1_in, read_data_1_out,

read_data_2_in, read_data_2_out,

sign_extended_immi_in, sign_extended_immi_out,

instruction_in, instruction_out);

// WB control signal

input RegWrite_in, MemtoReg_in;

output reg RegWrite_out, MemtoReg_out;

// MEM control signal

input Branch_in, MemRead_in, MemWrite_in, Jump_in;

output reg Branch_out, MemRead_out, MemWrite_out, Jump_out;

// EX control signal

input RegDst_in, ALUSrc_in;

output reg RegDst_out, ALUSrc_out;

input [1:0] ALUOp_in;

output reg [1:0] ALUOp_out;

// addr content

input [31:0] PC_plus4_in;

output reg [31:0] PC_plus4_out;

// data content

input [31:0] read_data_1_in, read_data_2_in, sign_extended_immi_in;

output reg [31:0] read_data_1_out, read_data_2_out, sign_extended_immi_out;

// reg content

input [31:0] instruction_in;

output reg [31:0] instruction_out;

// general signal

input clk, reset;

always @(posedge clk or posedge reset) begin

if (reset) begin

RegWrite_out <= 1'b0; MemtoReg_out <= 1'b0;

Branch_out <= 1'b0; MemRead_out <= 1'b0;

MemWrite_out <= 1'b0; Jump_out <= 1'b0;

RegDst_out <= 1'b0; ALUSrc_out <= 1'b0;

ALUOp_out <= 2'b0;

PC_plus4_out <= 32'b0;

read_data_1_out <= 32'b0; read_data_2_out <= 32'b0;

sign_extended_immi_out <= 32'b0;

instruction_out <= 32'b0;

end

else begin

RegWrite_out <= RegWrite_in; MemtoReg_out <= MemtoReg_in;

Branch_out <= Branch_in; MemRead_out <= MemRead_in;

MemWrite_out <= MemWrite_in; Jump_out <= Jump_in;

RegDst_out <= RegDst_in; ALUSrc_out <= ALUSrc_in;

ALUOp_out <= ALUOp_in; PC_plus4_out <= PC_plus4_in;

sign_extended_immi_out <= sign_extended_immi_in;

read_data_1_out <= read_data_1_in;

read_data_2_out <= read_data_2_in;

instruction_out <= instruction_in;

end

end

endmodule

// EX/MEM stage register

module EX_MEM_Stage_Reg (clk, reset,

RegWrite_in, RegWrite_out, MemtoReg_in, MemtoReg_out,

Branch_in, Branch_out, MemRead_in, MemRead_out,

MemWrite_in, MemWrite_out, Jump_in, Jump_out,

// Branch_addr_in, Branch_addr_out,

ALU_zero_in, ALU_zero_out,

ALU_result_in, ALU_result_out, Read_data_2_in, Read_data_2_out,

RegisterRd_in, RegisterRd_out);

// WB control signal

input RegWrite_in, MemtoReg_in;

output reg RegWrite_out, MemtoReg_out;

// MEM control signal

input Branch_in, MemRead_in, MemWrite_in, Jump_in;

output reg Branch_out, MemRead_out, MemWrite_out, Jump_out;

// addr content

// input [31:0] Branch_addr_in;

// output reg [31:0] Branch_addr_out;

// data content

input ALU_zero_in;

output reg ALU_zero_out;

// results

input [31:0] ALU_result_in, Read_data_2_in;

output reg [31:0] ALU_result_out, Read_data_2_out;

// registers

input [4:0] RegisterRd_in;

output reg [4:0] RegisterRd_out;

// general signal

input clk, reset;

always @(posedge clk or posedge reset) begin

if (reset) begin

RegWrite_out <= 1'b0; MemtoReg_out <= 1'b0;

Branch_out <= 1'b0; MemRead_out <= 1'b0;

MemWrite_out <= 1'b0; Jump_out <= 1'b0;

//   Branch_addr_out <= 32'b0;

ALU_zero_out <= 1'b0;

ALU_result_out <= 32'b0; Read_data_2_out <= 32'b0;

RegisterRd_out <= 5'b0;

end

else begin

RegWrite_out <= RegWrite_in; MemtoReg_out <= MemtoReg_in;

Branch_out <= Branch_in; MemRead_out <= MemRead_in;

MemWrite_out <= MemWrite_in; Jump_out <= Jump_in;

//   Branch_addr_out <= Branch_addr_in;

ALU_zero_out <= ALU_zero_in;

ALU_result_out <= ALU_result_in; Read_data_2_out <= Read_data_2_in;

RegisterRd_out <= RegisterRd_in;

end

end

endmodule

// MEM/WB stage register

module MEM_WB_Stage_Reg (RegWrite_in, RegWrite_out,

MemtoReg_in, MemtoReg_out,

Data_memory_read_in, Data_memory_read_out,

ALU_result_in, ALU_result_out,

Write_Register_in, Write_Register_out, clk, reset);

// WB control signal

input RegWrite_in, MemtoReg_in;

output reg RegWrite_out, MemtoReg_out;

// data content

input [31:0] Data_memory_read_in, ALU_result_in;

output reg [31:0] Data_memory_read_out, ALU_result_out;

input [4:0] Write_Register_in;

output reg [4:0] Write_Register_out;

// general signal

input clk, reset;

always @(posedge clk or posedge reset) begin

if (reset) begin

RegWrite_out <= 1'b0; MemtoReg_out <= 1'b0;

Data_memory_read_out <= 32'b0;  ALU_result_out <= 32'b0;

Write_Register_out <= 5'b0;

end

else begin

RegWrite_out <= RegWrite_in; MemtoReg_out <= MemtoReg_in;

Data_memory_read_out <= Data_memory_read_in; ALU_result_out <= ALU_result_in;

Write_Register_out <= Write_Register_in;

end

end

endmodule

// PC

module Program_Counter (clk, reset, PC_in, PC_out, PCWrite);

input PCWrite;

input clk, reset;

input [31:0] PC_in;

output reg [31:0] PC_out;

always @ (posedge clk or posedge reset) begin

if (reset)

PC_out <= 0;

else if (PCWrite)

PC_out <= PC_in;

end

endmodule

// contains hard-code instructions

module Instruction_Memory (address, instruction, reset);

input reset;

input [31:0] address;

output [31:0] instruction;

reg [31:0] mem [7:0]; // 8 instructions

integer k;

// get instruction right away

assign instruction = mem[address[6:2]];

// Initial setup at reset posedge

always @(posedge reset) begin

for (k = 0; k < 8; k = k + 1) begin

mem[k] = 32'b0; // add $0 $0 $0

end

mem[0] = 32'b100011_00010_00001_0000000000000100; // lw $1, 4($2)

mem[1] = 32'b000000_00001_00101_00100_00000_100010; // sub $4, $1, $5

mem[2] = 32'b000000_00001_00111_00110_00000_100100; // and $6, $1, $7

mem[3] = 32'b000000_00001_01001_01000_00000_100101; // or $8, $1, $9

mem[4] = 32'b000100_00110_00000_1111111111111011; // beq $6, $0, Label (-5)

// mem[0] = 32'b000000_00011_00100_00010_00000_100000; // add $2, $3, $4

// mem[1] = 32'b000000_00011_00100_00001_00000_100010; // sub $1, $3, $4

// mem[2] = 32'b100011_00110_00101_0000000000000000; // lw $5, 0($6)

// mem[3] = 32'b000100_00011_00100_1111111111111100; // beq $3, $4, Label (-4)

end

endmodule

// 32-bit ALU for addition only

module ALU_add_only (input1, input2, add_out);

input [31:0] input1, input2;

output [31:0] add_out;

assign add_out=input1+input2;

endmodule

// N_bit_MUX for Usability

module N_bit_MUX (input0, input1, mux_out, control);

parameter N = 32;

input [N-1:0] input0, input1;

input control;

output [N-1:0] mux_out;

assign mux_out = control ? input1 : input0;

endmodule

// sync register file (write/read occupy half cycle each)

// write: on rising edge; data width 32 bit; address width 5 bit

// read: on falling edge; data width 32 bit; address width 5 bit

module Register_File (Read_Register_1, Read_Register_2,

Write_Register, Write_Data, Read_Data_1, Read_Data_2,

RegWrite, clk, reset);

input [4:0] Read_Register_1, Read_Register_2, Write_Register;

input [31:0] Write_Data;

input clk, reset, RegWrite;

output reg [31:0] Read_Data_1, Read_Data_2;

reg [31:0] mem [15:0];

integer k;

always @(posedge clk or posedge reset) begin

if (reset) begin

for (k = 0; k < 16; k = k + 1) begin

mem[k] <= 32'b0;

end

mem[1] <= 20;

mem[2] <= 8;

mem[5] <= 2;

mem[6] <= 0;

mem[7] <= 1;

mem[9] <= 3;

// mem[3] = 32'b0011;

// mem[4] = 32'b0011;

// mem[6] = 32'h0000_0040;

end

else if (RegWrite) begin

mem[Write_Register] <= Write_Data;

end

end

always @(*) begin

Read_Data_1 <= mem[Read_Register_1];

Read_Data_2 <= mem[Read_Register_2];

end

endmodule

// sign-extend the 16-bit input to the 32_bit output

module Sign_Extension (input_16, output_32);

input [15:0] input_16;

output [31:0] output_32;

assign output_32[15:0]  = input_16[15:0];

assign output_32[31:16] = input_16[15] ? 16'b1111_1111_1111_1111: 16'b0;

endmodule

// Control Path

module Control (OpCode, RegDst,  MemRead, MemtoReg, ALUOp, MemWrite, ALUSrc, RegWrite,

Jump, Branch, IF_Flush, ID_Flush, EX_Flush);

input [5:0] OpCode;

output [1:0] ALUOp;

output RegDst, MemRead, MemtoReg, MemWrite, ALUSrc, RegWrite;

output Jump, Branch;

output IF_Flush, ID_Flush, EX_Flush;

// 000000 : add, sub, and, or, slt

// 001000 : addi

// 100011 : lw

// 101011 : sw

// 000100 : beq

// 000010 : j

// 000000 (R-format)

assign RegDst=(~OpCode[5])&(~OpCode[4])&(~OpCode[3])&(~OpCode[2])&(~OpCode[1])&(~OpCode[0]);

// 000000 (R-format)

assign ALUOp[1]=(~OpCode[5])&(~OpCode[4])&(~OpCode[3])&(~OpCode[2])&(~OpCode[1])&(~OpCode[0]);

// 000100 (beq)

assign ALUOp[0]=(~OpCode[5])&(~OpCode[4])&(~OpCode[3])&(OpCode[2])&(~OpCode[1])&(~OpCode[0]);

// 100011 (lw), 101011 (sw)

assign ALUSrc=((OpCode[5])&(~OpCode[4])&(~OpCode[3])&(~OpCode[2])&(OpCode[1])&(OpCode[0]))  |

((OpCode[5])&(~OpCode[4])&(OpCode[3])&(~OpCode[2])&(OpCode[1])&(OpCode[0]));

// 100011 (lw)

assign MemRead=(OpCode[5])&(~OpCode[4])&(~OpCode[3])&(~OpCode[2])&(OpCode[1])&(OpCode[0]);

// 101011 (sw)

assign MemWrite=(OpCode[5])&(~OpCode[4])&(OpCode[3])&(~OpCode[2])&(OpCode[1])&(OpCode[0]);

// 100011 (lw)

assign MemtoReg=(OpCode[5])&(~OpCode[4])&(~OpCode[3])&(~OpCode[2])&(OpCode[1])&(OpCode[0]);

// 000000 (R-format), 001000 (addi), 001100, 100011 (lw)

assign RegWrite=((~OpCode[5])&(~OpCode[4])&(~OpCode[3])&(~OpCode[2])&(~OpCode[1])&(~OpCode[0]))|

((~OpCode[5])&(~OpCode[4])&(OpCode[3])&(~OpCode[2])&(~OpCode[1])&(~OpCode[0])) |

((OpCode[5])&(~OpCode[4])&(~OpCode[3])&(~OpCode[2])&(OpCode[1])&(OpCode[0]));

// 000100 (beq)

assign Branch=(~OpCode[5])&(~OpCode[4])&(~OpCode[3])&(OpCode[2])&(~OpCode[1])&(~OpCode[0]);

// 000010 (j)

assign Jump=(~OpCode[5])&(~OpCode[4])&(~OpCode[3])&(~OpCode[2])&(OpCode[1])&(~OpCode[0]);

// only jump

assign IF_Flush=(~OpCode[5])&(~OpCode[4])&(~OpCode[3])&(~OpCode[2])&(OpCode[1])&(~OpCode[0]);

// jump or branch

assign ID_Flush=(~OpCode[5])&(~OpCode[4])&(~OpCode[3])&(~OpCode[2])&(OpCode[1])&(~OpCode[0]) |

(~OpCode[5])&(~OpCode[4])&(~OpCode[3])&(OpCode[2])&(~OpCode[1])&(~OpCode[0]);

// Umm...

assign EX_Flush=0;

endmodule

module ALU_Control (ALUOp, f_code, operation_code);

input [1:0] ALUOp;

input [5:0] f_code;

output [3:0] operation_code;

assign operation_code[3]=0;

// 0  1   | x x x x x x branch => subtract

// 1  0   | x x 0 0 1 0 => R-type subtract

// 1  0   | x x 1 0 1 0 => R-type slt

assign operation_code[2]=((~ALUOp[1])&(ALUOp[0])) |

((ALUOp[1])&(~ALUOp[0])&(~f_code[3])&(~f_code[2])&(f_code[1])&(~f_code[0])) |

((ALUOp[1])&(~ALUOp[0])&(f_code[3])&(~f_code[2])&(f_code[1])&(~f_code[0]));

// 0  0   | x x x x x x lw or sw => add

// 0  1   | x x x x x x branch => subtract

// 1  0   | x x 0 0 0 0 => R-type add

// 1  0   | x x 0 0 1 0 => R-type subtract

// 1  0   | x x 1 0 1 0 => R-type slt

assign operation_code[1]=((~ALUOp[1])&(~ALUOp[0])) |

((~ALUOp[1])&(ALUOp[0]))  |

((ALUOp[1])&(~ALUOp[0])&(~f_code[3])&(~f_code[2])&(~f_code[1])&(~f_code[0])) |

((ALUOp[1])&(~ALUOp[0])&(~f_code[3])&(~f_code[2])&(f_code[1])&(~f_code[0]))  |

((ALUOp[1])&(~ALUOp[0])&(f_code[3])&(~f_code[2])&(f_code[1])&(~f_code[0]));

// ALU OP | f_code field

// 1  0   | x x 0 1 0 1 => R-type Or

// 1  0   | x x 1 0 1 0 => R-type slt

assign operation_code[0]=((ALUOp[1])&(~ALUOp[0])&(~f_code[3])&(f_code[2])&(~f_code[1])&(f_code[0])) |

((ALUOp[1])&(~ALUOp[0])&(f_code[3])&(~f_code[2])&(f_code[1])&(~f_code[0]));

endmodule

// 32-bit ALU

module ALU (input1, input2, alu_out, zero, control);

// TODO : negative number handling

input [31:0] input1, input2;

input [3:0] control;

output reg [31:0] alu_out;

output reg zero;

always @ (control or input1 or input2) begin

case (control)

// and

4'b0000: begin alu_out<=input1&input2; zero<=0; end

// or

4'b0001: begin alu_out<=input1|input2; zero<=0; end

// add

4'b0010: begin alu_out<=input1+input2; zero<=0; end

// subtract

4'b0110: begin

if(input1 == input2)

zero <= 1;

else

zero <= 0;

alu_out <= input1 - input2;

end

// slt

4'b0111: begin

zero <= 0;

if(input1 - input2 >= 32'h8000_0000)

alu_out <= 32'b1;

else

alu_out <= 32'b0;

end

default: begin

zero <= 0;

alu_out <= input1;

end

endcase

end

endmodule

// Referred Previous Memory Project

module Data_Memory (MemAddr, Write_Data, Read_Data, clk, reset, MemRead, MemWrite);

input clk, reset;

input MemRead, MemWrite;

input [7:0] MemAddr;

input [31:0] Write_Data;

output reg [31:0] Read_Data;

reg [31:0] mem [31:0];

integer k;

always @(*) begin

if (reset) begin

for (k = 0; k < 32; k = k + 1) begin

mem[k] = 32'b0;

end

// mem[0xC] = 30;

mem[3] = 30;

mem[12] = 32'b0001_1110;

mem[16] = 30;

end

else

if (MemRead && !MemWrite) begin

Read_Data = mem[MemAddr[7:2]];

end

else if (!MemRead && MemWrite) begin

mem[MemAddr[7:2]] = Write_Data;

end

else begin

Read_Data = 32'bx;

end

end

endmodule

module Forwarding_Unit (EX_RegisterRs, EX_RegisterRt,

MEM_RegisterRd, WB_RegisterRd,

EX_MEM_RegWrite, MEM_WB_RegWrite,

// IF_ID_RegisterRs, IF_ID_RegisterRt,

ForwardA, ForwardB);

// ForwardC, ForwardD);

input [4:0] MEM_RegisterRd, WB_RegisterRd, EX_RegisterRs, EX_RegisterRt;

// input [4:0] IF_ID_RegisterRs,IF_ID_RegisterRt;

input EX_MEM_RegWrite, MEM_WB_RegWrite;

// output ForwardC,ForwardD;

output reg [1:0] ForwardA, ForwardB;

// reg ForwardC,ForwardD;

wire EX_HazardA, EX_HazardB;

wire WB_HazardA, WB_HazardB;

assign EX_HazardA = EX_MEM_RegWrite & (MEM_RegisterRd!=0) & (MEM_RegisterRd==EX_RegisterRs);

assign EX_HazardB = EX_MEM_RegWrite & (MEM_RegisterRd!=0) & (MEM_RegisterRd==EX_RegisterRt);

assign WB_HazardA = MEM_WB_RegWrite & (WB_RegisterRd!=0) & (WB_RegisterRd==EX_RegisterRs);

assign WB_HazardB = MEM_WB_RegWrite & (WB_RegisterRd!=0) & (WB_RegisterRd==EX_RegisterRt);

always@ (EX_MEM_RegWrite or MEM_WB_RegWrite or EX_HazardA or WB_HazardA or EX_HazardB or WB_HazardB) begin

if(EX_HazardA)

ForwardA<=2'b10;

else if(!EX_HazardA & WB_HazardA)

ForwardA<=2'b01;

else

ForwardA<=2'b00;

if(EX_HazardB)

ForwardB<=2'b10;

else if(!EX_HazardB & WB_HazardB)

ForwardB<=2'b01;

else

ForwardB<=2'b00;

end

endmodule

module Hazard_Detection_Unit (ID_EX_RegisterRt, ID_EX_MemRead,

IF_ID_RegisterRs, IF_ID_RegisterRt,

PCWrite, IF_ID_Write, Hazard);

input [4:0] IF_ID_RegisterRs, IF_ID_RegisterRt;

input [4:0] ID_EX_RegisterRt;

input ID_EX_MemRead;

output reg PCWrite, IF_ID_Write, Hazard;

wire Rs_Stall, Rt_Stall;

assign Rs_Stall=(ID_EX_RegisterRt==IF_ID_RegisterRs);

assign Rt_Stall=(ID_EX_RegisterRt==IF_ID_RegisterRt);

always@(*) begin

if(ID_EX_MemRead & (Rs_Stall|Rt_Stall)) begin

PCWrite <= 0;

IF_ID_Write <=0;

Hazard <= 1;

end

else begin

PCWrite <= 1;

IF_ID_Write <= 1;

Hazard <= 0;

end

end

endmodule

module Reset_Control (RegDst_in, ALUSrc_in, Branch_in, MemRead_in,

MemtoReg_in, MemWrite_in, RegWrite_in, Control_Reset,

RegDst_out, ALUSrc_out, Branch_out, MemRead_out,

MemtoReg_out, MemWrite_out, RegWrite_out);

input RegDst_in, ALUSrc_in, Branch_in, MemRead_in;

input MemtoReg_in, MemWrite_in, RegWrite_in;

input Control_Reset;

output RegDst_out, ALUSrc_out, Branch_out, MemRead_out;

output MemtoReg_out, MemWrite_out, RegWrite_out;

assign RegDst_out = Control_Reset ? 0 : RegDst_in;

assign ALUSrc_out = Control_Reset ? 0 : ALUSrc_in;

assign Branch_out = Control_Reset ? 0 : Branch_in;

assign MemRead_out = Control_Reset ? 0 : MemRead_in;

assign MemtoReg_out = Control_Reset ? 0 : MemtoReg_in;

assign MemWrite_out = Control_Reset ? 0 : MemWrite_in;

assign RegWrite_out = Control_Reset ? 0 : RegWrite_in;

endmodule

module Mux_32bit_3to1 (in00, in01, in10, mux_out, control);

input [31:0] in00, in01, in10;

output [31:0] mux_out;

input [1:0] control;

reg [31:0] mux_out;

always @(in00 or in01 or in10 or control)

begin

case(control)

2'b00:mux_out<=in00;

2'b01:mux_out<=in01;

2'b10:mux_out<=in10;

default: mux_out<=in00;

endcase

end

endmodule

2. 실행 결과

위 코드는 상단 조건에 따라 다음과 같은 작업을 반복한다.

Beq에 의해 다시 Label로 되돌아가기 때문에, 같은 과정이 반복된다.

| PC Value | Fetch | Decode | Execute | Memory | Write Back |
| --- | --- | --- | --- | --- | --- |
| 00000 | lw |  |  |  |  |
| 00100 | Sub | lw |  |  |  |
| 01000 | And | Sub | lw |  |  |
| 01000 | And | Sub | Stall | lw |  |
| 01100 | Or | And | Sub(Forwarding) | Garbage | lw |
| 10000 | Beq | Or | And | Sub | Garbage |
| 10100 | Flush | Beq | Or | And | Sub |
| 00000(반복) | lw | NOP | Beq | Or | And |
| 00100 | Sub | Lw | NOP | Beq | Or |
| 01000 | Or | Sub | Lw | NOP | Beq |
| 01000 | Beq | Sub | Stall | lw | NOP |

Advanced Pipeline으로 변경되면서 논점이 되는 포인트들은 다음과 같다.

- 01000에서, lw다음으로 sub이 오기 때문에, stall이 발생한다는 것을 알 수 있다.

- 01100에서, sub의 rs인 $1의 값이 lw로부터 forwarding된다는 것을 알 수 있다.

- 10100에서, Beq에 의한 Flush가 발생한다는 것을 알 수 있다.

디버깅을 위한 매개변수들을 추가하여 만든 waveform은 다음과 같다.

각 매개변수들에 대한 설명은 다음과 같다.

- Result : ALU 계산 결과 (후에 FPGA LED 값이 된다.)

- PC_Now : 현 시점에서 Instruction Fetch에서 사용되는 PC를 나타냄

- Instruction_Now : 현 시점에서 Instruction Decode에서 사용되는 Instruction을 나타냄

- Rs_Now & Rt_Now : Instruction_Now를 통해 Register_File에서 가져온 Rs,Rt 값이다. Falling Edge에서 최신화됨

- ALU_Input_1, ALU_Input_2 : Execute 단계에서 ALU의 input이 되는 두 피연산자

- Load_data : Memory 단계에서 읽어온 데이터 (현재는 lw에서만 해당)

- Beq_Address : Execute 단계에서 계산되는 branch address

- J_address : Execute 단계에서 계산되는 Jump할 Address

- Immi_Shifted : Execute 단계에서 계산되는 32비트 shift된 값 (이것이 PC와 합쳐져 Beq_Address가 된다.)

- PC_4 : Execute 단계에서의 PC, 큰 의미는 없다. 편의를 위해 추가

- Debug_flag_2 : 편의상 이것저것을 확인하기 위한 flag, 큰 의미는 없다.

- Debug_Write_Data, Write_Register : Decode 단계의 Register_File에 Write되는 Register Address와 Data

- Debug_RegWrite, Debug_PCWrite, Debug_IF_ID_Write, Debug_IF_Flush : Hazard Detection과 stall 기능이 추가됨에 따른 Control 비트를 디버깅한다.

- Debug_ForWardA, Debug_ForWardB : Forwarding Unit이 추가됨에 따른 2비트의 Forwarding A, Forwarding B를 디버깅한다.

| lw | sub |
| --- | --- |
| And | Sub Stall |
| Or | Beq |
| NOP | lw (반복) |
| Sub (반복) |  |

각 Pipeline 단계별로 결과를 구조화해보면 다음과 같다.

| PC_Now | Fetch | Decode | Execute | Memory | Write Back |
| --- | --- | --- | --- | --- | --- |
| 00000 | Instruction_Now = lw |  |  |  |  |
| 00100 | Instruction_Now = Sub | Rs=$2 / Rt=$1 |  |  |  |
| 01000 | Instruction_Now = And | Rs=$1 / Rt=$5 | Result=$2+4=1100 |  |  |
| 01000 | Instruction_Now = And | Rs=$1 / Rt=$5 | Stall (쓰레기 값) | Mem[0xC] => 30 |  |
| 01100 | Instruction_Now = Or | Rs=$1 / Rt=$7 | Result=$1-$5=28 | Stall | $1에 결과 30저장요청 |
| 10000 | Instruction_Now = Beq | Rs=$1 / Rt=$9 | Result=$1 & $5=0 WB Forwarding 발생 | sub이므로 없음 | Stall |
| 10100 | Flush | Rs=$6 / Rt=$0 Branch주소계산 | Result=30 ¦ 3 =11111 | And이므로 없음 |  |
| 00000(반복 | Instruction_Now = lw |  | Result=$6-$0=0 | Or이므로 없음 | $6에 결과 0 저장요청 |
| 00100 | Instruction_Now = Sub | Rs=$2 / Rt=$1 |  | Beq이므로 없음 | $8에 결과 11111 저장 |
| 01000 | Instruction_Now = And | Rs=$1 / Rt=$5 | Result=$2+4=1100 |  | Beq이므로 없음 |
| 01000 | Instruction_Now = And | Rs=$1 / Rt=$5 | Stall (쓰레기 값) | Mem[0xC] => 30 |  |

3. 코드 설명

필요하다고 판단되는 Unit에 대한 코드 설명을 추가한다.

- Forwarding Unit

module Forwarding_Unit (EX_RegisterRs, EX_RegisterRt,

MEM_RegisterRd, WB_RegisterRd,

EX_MEM_RegWrite, MEM_WB_RegWrite,

// IF_ID_RegisterRs, IF_ID_RegisterRt,

ForwardA, ForwardB);

// ForwardC, ForwardD);

input [4:0] MEM_RegisterRd, WB_RegisterRd, EX_RegisterRs, EX_RegisterRt;

// input [4:0] IF_ID_RegisterRs,IF_ID_RegisterRt;

input EX_MEM_RegWrite, MEM_WB_RegWrite;

// output ForwardC,ForwardD;

output reg [1:0] ForwardA, ForwardB;

// reg ForwardC,ForwardD;

wire EX_HazardA, EX_HazardB;

wire WB_HazardA, WB_HazardB;

assign EX_HazardA = EX_MEM_RegWrite & (MEM_RegisterRd!=0) & (MEM_RegisterRd==EX_RegisterRs);

assign EX_HazardB = EX_MEM_RegWrite & (MEM_RegisterRd!=0) & (MEM_RegisterRd==EX_RegisterRt);

assign WB_HazardA = MEM_WB_RegWrite & (WB_RegisterRd!=0) & (WB_RegisterRd==EX_RegisterRs);

assign WB_HazardB = MEM_WB_RegWrite & (WB_RegisterRd!=0) & (WB_RegisterRd==EX_RegisterRt);

always@ (EX_MEM_RegWrite or MEM_WB_RegWrite or EX_HazardA or WB_HazardA or EX_HazardB or WB_HazardB) begin

if(EX_HazardA)

ForwardA<=2'b10;

else if(!EX_HazardA & WB_HazardA)

ForwardA<=2'b01;

else

ForwardA<=2'b00;

if(EX_HazardB)

ForwardB<=2'b10;

else if(!EX_HazardB & WB_HazardB)

ForwardB<=2'b01;

else

ForwardB<=2'b00;

end

endmodule

00, 01, 10의 세가지 결과를 갖게 되며, 각 상황에 부합하는 조건은 교과서의 다음 로직을 따른다.

Forwarding Unit의 결과에 따라 ALU의 input이 달라지며, 이는 다음과 같이 구현 가능하다.

Mux_32bit_3to1 ALU_MuxA (.in00(EX_Read_data_1), .in01(WB_Write_Data), .in10(MEM_ALU_result),

.mux_out(muxA_out), .control(ForwardA));

Mux_32bit_3to1 ALU_MuxB (.in00(EX_Read_data_2), .in01(WB_Write_Data), .in10(MEM_ALU_result),

.mux_out(muxB_out), .control(ForwardB));

// assign Debug_ForWardA = ForwardA;

// assign Debug_ForWardB = ForwardB;

N_bit_MUX #(32) alu_input_mux (.input0(muxB_out), .input1(EX_sign_extended_immi),

.mux_out(ALU_input_B), .control(EX_ALUSrc));

- Hazard Detection Unit

module Hazard_Detection_Unit (ID_EX_RegisterRt, ID_EX_MemRead,

IF_ID_RegisterRs, IF_ID_RegisterRt,

PCWrite, IF_ID_Write, Hazard);

input [4:0] IF_ID_RegisterRs, IF_ID_RegisterRt;

input [4:0] ID_EX_RegisterRt;

input ID_EX_MemRead;

output reg PCWrite, IF_ID_Write, Hazard;

wire Rs_Stall, Rt_Stall;

assign Rs_Stall=(ID_EX_RegisterRt==IF_ID_RegisterRs);

assign Rt_Stall=(ID_EX_RegisterRt==IF_ID_RegisterRt);

always@(*) begin

if(ID_EX_MemRead & (Rs_Stall|Rt_Stall)) begin

PCWrite <= 0;

IF_ID_Write <=0;

Hazard <= 1;

end

else begin

PCWrite <= 1;

IF_ID_Write <= 1;

Hazard <= 0;

end

end

endmodule

lw의 결과를 바로 다음 연산에서 사용하고자 하는 경우 발생하며, 그 조건은 수업 중 살펴본 바와 같이 다음과 같다.

If (IF.ID.RS or IF.ID.RT == ID.EXE.RT) and (MemRead == 1)

IF.ID.Write를 0으로 만들고 PCWrite도 0으로 만든다. 그리고 Hazard라는 wire를 1로 하는데, 이것은 Control 신호를 제어하는데 사용한다.

Hazard_Detection_Unit hazard_detection_unit (.ID_EX_RegisterRt(EX_instruction[20:16]), .ID_EX_MemRead(EX_MemRead),

.IF_ID_RegisterRs(ID_instruction[25:21]), .IF_ID_RegisterRt(ID_instruction[20:16]),

.PCWrite(PCWrite), .IF_ID_Write(IF_ID_Write), .Hazard(Hazard));

Control control_unit (.OpCode(ID_instruction[31:26]),

.RegDst(RegDst), .Jump(Jump), .Branch(Branch),

.MemRead(MemRead), .MemtoReg(MemtoReg), .ALUOp(ALUOp),

.MemWrite(MemWrite), .ALUSrc(ALUSrc), .RegWrite(RegWrite),

.IF_Flush(IF_Flush), .ID_Flush(ID_Flush), .EX_Flush(EX_Flush));

or (Control_Reset, ID_Flush, Hazard);

Reset_Control reset_control_unit(

.RegDst_in(RegDst), .ALUSrc_in(ALUSrc), .Branch_in(Branch),

.MemRead_in(MemRead), .MemtoReg_in(MemtoReg),

.MemWrite_in(MemWrite), .RegWrite_in(RegWrite),

.RegDst_out(RegDst_t), .ALUSrc_out(ALUSrc_t), .Branch_out(Branch_t),

.MemRead_out(MemRead_t), .MemtoReg_out(MemtoReg_t),

.MemWrite_out(MemWrite_t), .RegWrite_out(RegWrite_t),

.Control_Reset(Control_Reset));

위 코드에서 볼 수 있듯, Hazard와 Control로부터의 ID_Flush값들 중 하나라도 1이 된다면 Control과 관련된 wire를 전부 0으로 만드는 Reset_Control이 실행된다. 그래서, lw 다음의 Sub가 ALU연산을 할 지 모르지만, 메모리에는 어떠한 영향도 주지 않기 때문에, 사실상 쓰레기 값이 된다.

다음으로 Branch에 따른 Instruction Fetch 단계에서의 변화가 있다.

and (Branch_taken, Branch, Rs_Rt_equal);

N_bit_MUX #(32) branch_mux (.input0(PC_plus4), .input1(ID_Branch_addr),

.mux_out(Branch_MUX_output), .control(Branch_taken));

N_bit_MUX #(32) jump_mux (.input0(Branch_MUX_output), .input1(Jump_Address),

.mux_out(PC_in), .control(Jump));

과제 그림에서 볼 수 있듯이, RS와 RT가 같고, Control output 중 Branch가 1이라면, 추가 Step을 밟지 않고 바로 Branch 및 Flush를 진행한다. 위 그림은 Jump에 대한 경우도 보이고 있는데, 이것은 후에 설명하겠다.

// IF/ID stage register

module IF_ID_Stage_Reg (clk, reset,

PC_plus4_in, PC_plus4_out,

instruction_in, instruction_out,

IF_ID_Write, IF_Flush);

input clk, reset;

input [31:0] PC_plus4_in, instruction_in;

output reg [31:0] PC_plus4_out, instruction_out;

// Hazard

input IF_Flush, IF_ID_Write;

always @(posedge clk or negedge reset) begin

if (!reset) begin

PC_plus4_out <= 32'b0;

instruction_out <= 32'b0;

end

else if (IF_Flush) begin

// TODO : Check condition

// PC_plus4_out <= 32'b0;

// PC_plus4_out <= PC_plus4_in;

instruction_out <= 32'b0;

end

else if (IF_ID_Write) begin

PC_plus4_out <= PC_plus4_in;

instruction_out <= instruction_in;

end

end

endmodule

- IF_ID_Stage_Reg

IF.ID.Write와 IF.Flush가 추가됨에 따라 이에 대한 추가 처리가 이루어진다.

IF.Flush는 instruction을 전부 0으로 만들고, IF.ID.Write는 0이 될 시 PC update가 이루어지지 않는다.

다음으로 FPGA 구동을 위한 Startup 코드를 보자.

- Startup

`timescale 1ns / 1ps

//////////////////////////////////////////////////////////////////////////////////

// Company:

// Engineer:

//

// Create Date:    23:36:42 06/21/2021

// Design Name:

// Module Name:    startup

// Project Name:

// Target Devices:

// Tool versions:

// Description:

//

// Dependencies:

//

// Revision:

// Revision 0.01 - File Created

// Additional Comments:

//

//////////////////////////////////////////////////////////////////////////////////

module Startup(

input   clk_50MHz,

input reset,

output [15:0] ALU_Result,

output reg [3:0] digit,

output reg [7:0] fnd,

output Branch,

output Jump,

output RS_RT_Equal,

output Branch_Taken

);

reg [24:0] counter;

reg [3:0] number;

reg [15:0] test_led = 0;

wire clk_slow;

reg digit_sel;

reg clk_operating;

assign clk_slow = clk_operating;

wire slow_clock;

wire [31:0] PC;

clock_divider my_divider(.clk(clk_50MHz), .rst(reset), .clk_operating(slow_clock), .my_clk(clk));

//assign debug_slow_clock = slow_clock;

advanced_pipeline my_pipeline(.clk(slow_clock), .reset(reset), .result(ALU_Result), .PC_now(PC),

.d_Branch(Branch), .d_Jump(Jump), .d_RS_RT_Equal(RS_RT_Equal), .d_Branch_Taken(Branch_Taken));

//assign Debug_ALU = ALU_Result;

//Fnd number

// always @(posedge clk or negedge reset)

always @(posedge clk or posedge reset)

begin

if (reset)

begin

digit_sel<=0;

digit<=4'b0000;

number<=0;

end

else

begin

digit_sel <= digit_sel + 1'b1;

case(digit_sel)

0: begin digit<=4'b1110; number<= PC % 32'd10; end

1: begin digit<=4'b1101; number<= PC / 32'd10; end

endcase

end

end

//Fnd number

always @(*)

case (number)

4'h0 : fnd=8'b00000011;

4'h1 : fnd=8'b10011111;

4'h2 : fnd=8'b00100101;

4'h3 : fnd=8'b00001101;

4'h4 : fnd=8'b10011001;

4'h5 : fnd=8'b01001001;

4'h6 : fnd=8'b01000001;

4'h7 : fnd=8'b00011011;

4'h8 : fnd=8'b00000001;

4'h9 : fnd=8'b00011001;

4'ha : fnd=8'b10001001;

4'hb : fnd=8'b10000011;

4'hc : fnd=8'b01100011;

4'hd : fnd=8'b10000101;

4'he : fnd=8'b01100001;

4'hf : fnd=8'b01110001;

default : fnd=8'b00000000;

endcase

endmodule

진행중인 PC값의 변화를 살피기 위해 fnd 에 이를 표시하는 코드가 추가되었다.

이는 동기인 이주영 학우로부터 도움을 받았다.

clock_divider my_divider(.clk(clk_50MHz), .rst(reset), .clk_operating(slow_clock), .my_clk(clk));

advanced_pipeline my_pipeline(.clk(slow_clock), .reset(reset), .result(ALU_Result), .PC_now(PC),

.d_Branch(Branch), .d_Jump(Jump), .d_RS_RT_Equal(RS_RT_Equal), .d_Branch_Taken(Branch_Taken));

느린 counter를 만들기 위해, 수업 중 제시되었던 clock_divider를 사용하고, 이것을 pipeline의 input으로 입력하였다.

구현 중 각 Stage를 넘어가면서 non-blocking으로 구현을 하지 않아 많은 시간을 디버깅에 할애하였다.

LED의 결과, 즉 ALU의 결과는 다음 값들을 반복한다.

상단의 표 중에서 Execute단만을 가져왔으며, 결과는 2진수로 LED 위에 표현될 것이다.

| PC_Now | Execute |
| --- | --- |
| 00000 | All Zero |
| 00100 | All Zero |
| 01000 | 1100 |
| 01000 | 1010 |
| 01100 | 11100 |
| 10000 | 00000 |
| 10100 | 11111 |
| 00000 | 00000 (반복) |
| 00100 | 00000 |

첨부하는 추가 TB 파일들은 다음과 같다.

tb_forwarding_unit : forwarding unit을 위한 test bench

tb_hazard_control : hazard detection unit을 위한 test bench